SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE:CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Code of Conduct:

I  LAKSHA.M P (1924260302) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: Laksha. M P

| Criteria | Weightage (%) | Q1 10marks | Q2 10marks | Q3 10marks | Q4 10marks |
| --- | --- | --- | --- | --- | --- |
| Understanding of Scenario | 20% |  |  |  |  |
| Identification of Key Issues | 20% |  |  |  |  |
| Application of Concepts | 25% |  |  |  |  |
| Decision Making | 20% |  |  |  |  |
| Clarity of Response | 15% |  |  |  |  |

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid host IP address range for each subnet.

- Calculate the number of usable host addresses available in each subnet.

- Determine the remaining unused IP address range within the given network block.

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid range of host IP addresses.

- Calculate the number of usable hosts in each subnet.

- Identify the unused IP address range remaining within the allocated IP block.

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- the total number of possible host addresses available within this network.

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

ANSWERS – SCENARIO-BASED ASSESSMENT

Question 1: Smart Campus Subnet Allocation

Given network: 192.168.100.0/24. The subnets are allocated sequentially from the beginning of the address block.

1. School of Computing – 192.168.100.0/25

Network address: 192.168.100.0

Broadcast address: 192.168.100.127

Valid host range: 192.168.100.1 to 192.168.100.126

Usable hosts: 2^(32−25) − 2 = 128 − 2 = 126

2. School of Electronics – 192.168.100.128/26

Network address: 192.168.100.128

Broadcast address: 192.168.100.191

Valid host range: 192.168.100.129 to 192.168.100.190

Usable hosts: 2^(32−26) − 2 = 64 − 2 = 62

3. School of Mechanical Engineering – 192.168.100.192/27

Network address: 192.168.100.192

Broadcast address: 192.168.100.223

Valid host range: 192.168.100.193 to 192.168.100.222

Usable hosts: 2^(32−27) − 2 = 32 − 2 = 30

4. Remaining unused range: 192.168.100.224 to 192.168.100.255. This range contains 32 addresses (30 usable host addresses if treated as a /27 subnet).

Question 2: Company Department Subnet Allocation

Given network block: 10.10.0.0/16. The subnets are allocated sequentially from the beginning of the block.

1. Software Development – 10.10.0.0/24

Network address: 10.10.0.0; broadcast address: 10.10.0.255

Valid host range: 10.10.0.1 to 10.10.0.254

Usable hosts: 2^(32−24) − 2 = 254

2. Quality Assurance – 10.10.1.0/25

Network address: 10.10.1.0; broadcast address: 10.10.1.127

Valid host range: 10.10.1.1 to 10.10.1.126

Usable hosts: 2^(32−25) − 2 = 126

3. Human Resources – 10.10.1.128/26

Network address: 10.10.1.128; broadcast address: 10.10.1.191

Valid host range: 10.10.1.129 to 10.10.1.190

Usable hosts: 2^(32−26) − 2 = 62

4. Executive Management – 10.10.1.192/27

Network address: 10.10.1.192; broadcast address: 10.10.1.223

Valid host range: 10.10.1.193 to 10.10.1.222

Usable hosts: 2^(32−27) − 2 = 30

Unused range in the original /16 block: 10.10.1.224 to 10.10.255.255. The four assigned subnets use 480 addresses in total; the remaining 65,056 addresses are unallocated.

Question 3: IPv6 Addressing

Given IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

1. Compressed notation: 2001:db8:abcd::/64

2. Full hexadecimal representation: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

3. Network prefix: 2001:0db8:abcd:0000::/64. The first 64 bits (the first four hextets) identify the network prefix.

4–5. A /64 prefix leaves 64 bits for interface identifiers. Therefore, the subnet contains 2^64 = 18,446,744,073,709,551,616 possible interface addresses. IPv6 does not use the IPv4-style network and broadcast address subtraction; the exact number of addresses available for host/interface assignment can depend on network practices.

Question 4: Routing Table and Packet Forwarding

Destination IP address: 192.168.2.45

1. Destination subnet: 192.168.2.0/24, because the first 24 bits match that subnet.

2. Matching routing table entry: 192.168.2.0/24.

3. The router forwards the packet through the interface associated with the 192.168.2.0/24 route, or to that route's configured next hop. The question does not provide interface names or next-hop IP addresses, so a specific interface/IP cannot be determined from the given information.

4. If no specific routing-table entry matches the destination, the router uses the default route 0.0.0.0/0 if one is configured. If no default route exists, the packet is normally dropped.

Question 5: Administration and Finance LANs

1. Administration LAN – 192.168.10.0/26

Network address: 192.168.10.0

Broadcast address: 192.168.10.63

Valid host range: 192.168.10.1 to 192.168.10.62

2. Finance LAN – 192.168.10.64/26

Network address: 192.168.10.64

Broadcast address: 192.168.10.127

Valid host range: 192.168.10.65 to 192.168.10.126

Each /26 subnet has 2^(32−26) − 2 = 62 usable host addresses.

3. Example computer addresses:

Administration computers: 192.168.10.2, 192.168.10.3, and 192.168.10.4.

Finance computers: 192.168.10.66, 192.168.10.67, and 192.168.10.68.

4. Default gateways:

Administration LAN: 192.168.10.1

Finance LAN: 192.168.10.65

Each computer should use the gateway configured for its own LAN to communicate with devices on other networks.