SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software: Cisco Packet Tracer, Wireshark

Code of Conduct:

I  LAKSHA.M P (1924260302) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: LAKSHA.MP

| Criteria | Weightage | Q1 10marks | Q2 10marks | Q3 10marks | Q4 10marks |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% |  |  |  |  |
| Debugging | 25% |  |  |  |  |
| Optimization | 20% |  |  |  |  |
| Analysis | 20% |  |  |  |  |
| Code Quality | 15% |  |  |  |  |

Annexure A

Debugging & Optimisation Task

Name: Laksha.M P  |   Reg No: 1924260302  |   Course: CSA07 Computer Networks

Question 1: Workstation 192.168.10.65/26, gateway 192.168.10.1

Given: IP 192.168.10.65, mask 255.255.255.192 (/26), gateway 192.168.10.1. The host cannot reach systems in 192.168.10.0/26.

## 1. Subnet of the configured IP

/26 gives a block size of 256 − 192 = 64, so the subnets in 192.168.10.0/24 are .0, .64, .128 and .192. The address 192.168.10.65 lies in 192.168.10.64/26.

## 2. Network, broadcast and host range

| Subnet | Network | Broadcast | Valid host range | Usable |
| --- | --- | --- | --- | --- |
| 192.168.10.0/26 | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 | 62 |
| 192.168.10.64/26 (workstation) | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 | 62 |

## 3. Validity of the IP and gateway

- IP 192.168.10.65: valid. It is the first usable host of 192.168.10.64/26 (not the network or broadcast address).

- Gateway 192.168.10.1: valid host, but it belongs to 192.168.10.0/26, a different subnet from the workstation. It is not directly reachable, so it is invalid for this host.

## 4. Configuration errors

- The workstation sits in 192.168.10.64/26, while the systems it must reach are in 192.168.10.0/26. Two different subnets need a router, and the host has no valid route.

- The default gateway is outside the host's own subnet. A host only ARPs for a gateway inside its own network, so the gateway is never reached and every packet to another subnet is lost.

- Root cause: the host IP and the gateway were taken from different /26 blocks.

## 5. Optimized configuration

Move the host into the target subnet 192.168.10.0/26 and keep the existing gateway:

| Parameter | Corrected value |
| --- | --- |
| IP address | 192.168.10.10 |
| Subnet mask | 255.255.255.192 (/26) |
| Default gateway | 192.168.10.1 |

(Any free address from 192.168.10.2 to 192.168.10.62 works. 192.168.10.1 is reserved for the router.)

## 6. Usable hosts after correction

Host bits = 32 − 26 = 6, so usable hosts = 2^6 − 2 = 64 − 2 = 62 (192.168.10.1 to 192.168.10.62, of which one is used by the gateway).

Question 2: IPv6 devices A and B

Given: Device A 2001:db8::ff00:42:8329/64 and Device B 2001:db8:0:0:1::1/64. Both are on the same link but cannot communicate.

## 1. Full (expanded) notation

| Device | Compressed | Expanded |
| --- | --- | --- |
| A | 2001:db8::ff00:42:8329 | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| B | 2001:db8:0:0:1::1 | 2001:0db8:0000:0000:0001:0000:0000:0001 |

In A, "::" replaces three zero groups. In B, "::" replaces two zero groups (groups 6 and 7).

## 2. Network prefix (first 64 bits = first 4 groups)

- Device A: 2001:0db8:0000:0000::/64 = 2001:db8::/64

- Device B: 2001:0db8:0000:0000::/64 = 2001:db8::/64

## 3. Same subnet?

Yes. With a /64 mask both addresses give the identical prefix 2001:db8::/64, so they are in the same subnet. The fault is therefore not a different prefix length or a different network prefix.

## 4. Addressing or prefix mismatch

- Device B's interface ID is 0001:0000:0000:0001. The "1" in the fifth group sits inside the interface-ID half, not in the subnet-ID field. This looks like a typing slip: the intended address was almost certainly 2001:db8:0:1::1 (subnet ID 1). Device B is placed in the wrong place in the addressing plan, and its interface ID is non-standard and hard to track.

- Device A's interface ID is 0000:ff00:0042:8329, which is also arbitrary and follows no planned pattern.

- Since the prefixes already match, the remaining causes of failure must be checked on the devices: IPv6 not enabled or no link-local address, wrong VLAN or port, a duplicate address, or ICMPv6/ND being filtered. Verify with the commands below.

Router(config-if)# ipv6 address 2001:db8:0:1::1/64   ! gateway

Router(config)# ipv6 unicast-routing

PC> ipv6 config            ! check address, prefix, link-local

PC> ping 2001:db8:0:1::20   ! test reachability

Router# show ipv6 interface brief

Router# show ipv6 neighbors  ! confirm Neighbor Discovery works

## 5. Optimized addressing

| Device | Address | Prefix |
| --- | --- | --- |
| Router (gateway) | 2001:db8:0:1::1 | /64 |
| Device A | 2001:db8:0:1::10 | /64 |
| Device B | 2001:db8:0:1::20 | /64 |

All three share the prefix 2001:db8:0:1::/64, with a clear, sequential interface ID in the last group.

## 6. Available host addresses

A /64 leaves 128 − 64 = 64 interface-ID bits, so there are 2^64 = 18,446,744,073,709,551,616 addresses. IPv6 has no broadcast address, so every address is usable (the all-zeros subnet-router anycast address is conventionally kept aside, giving 2^64 − 1 for hosts).

Question 3: Subnetting 192.168.1.0/24 with /26, /27, /28

Note: the question does not list the departments or their sizes, so the allocation and host requirements below are assumed to match the problem described (some departments short of addresses, others with many unused). The method applies to any real figures.

## 1. Possible subnets by mask

| Mask | Block size | Subnets in /24 | Usable hosts each | Network addresses (last octet) |
| --- | --- | --- | --- | --- |
| /26 | 64 | 4 | 62 | 0, 64, 128, 192 |
| /27 | 32 | 8 | 30 | 0, 32, 64, 96, 128, 160, 192, 224 |
| /28 | 16 | 16 | 14 | 0, 16, 32, 48, 64, 80, 96, 112, 128, 144, 160, 176, 192, 208, 224, 240 |

## 2. Assumed current allocation and requirements

| Dept | Mask | Network | Broadcast | Host range | Usable | Needed | Unused / short |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Engineering | /26 | 192.168.1.0 | 192.168.1.63 | .1 – .62 | 62 | 50 | 12 unused |
| HR | /27 | 192.168.1.64 | 192.168.1.95 | .65 – .94 | 30 | 10 | 20 unused |
| Sales | /28 | 192.168.1.80 | 192.168.1.95 | .81 – .94 | 14 | 25 | 11 short |
| Admin | /28 | 192.168.1.96 | 192.168.1.111 | .97 – .110 | 14 | 12 | 2 unused |

## 3. Inefficient allocations

- HR /27: 30 usable for 10 hosts, so 20 addresses (about 67%) are wasted. A /28 is enough.

- Sales /28: only 14 usable for 25 hosts, so it is 11 addresses short. It needs a /27.

- Engineering /26: 12 spare addresses (about 19%). This is acceptable and allows growth.

- Admin /28: 2 spare, a good fit.

## 4. Allocation mistakes found

- Overlap: Sales 192.168.1.80/28 (.80 – .95) lies entirely inside HR 192.168.1.64/27 (.64 – .95). Overlapping subnets cause routing ambiguity and duplicate-address problems.

- Wrong mask sizing: subnets were chosen without matching masks to host counts (the big mask to the small department, the small mask to the large department).

- No planning for WAN links: router-to-router links would waste a full /28 or /27 each if added later.

## 5. Optimized VLSM scheme

Method: sort requirements from largest to smallest and allocate each at the next free block boundary, using the smallest mask that fits.

| Dept | Hosts needed | Mask | Network | Broadcast | Host range | Usable | Spare |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Engineering | 50 | /26 | 192.168.1.0 | 192.168.1.63 | .1 – .62 | 62 | 12 |
| Sales | 25 | /27 | 192.168.1.64 | 192.168.1.95 | .65 – .94 | 30 | 5 |
| HR | 10 | /28 | 192.168.1.96 | 192.168.1.111 | .97 – .110 | 14 | 4 |
| Admin | 12 | /28 | 192.168.1.112 | 192.168.1.127 | .113 – .126 | 14 | 2 |
| WAN link 1 | 2 | /30 | 192.168.1.128 | 192.168.1.131 | .129 – .130 | 2 | 0 |
| WAN link 2 | 2 | /30 | 192.168.1.132 | 192.168.1.135 | .133 – .134 | 2 | 0 |

Free space for growth: 192.168.1.136 – 192.168.1.255 (120 addresses), one contiguous block, so no overlap and nothing wasted. Spare host addresses across the four departments change from 34 in the old plan (12 + 20 + 2, with Sales still 11 short) to 23 in the new plan (12 + 5 + 4 + 2), and every department now has enough addresses.

Check: Engineering .0–.63, Sales .64–.95, HR .96–.111, Admin .112–.127, WAN .128–.135. All blocks are aligned to their own size and do not overlap.

Question 4: IPv6 devices A and B

Given: Device A 2001:db8::ff00:42:8329/64 and Device B 2001:db8:0:0:1::1/64. Both are on the same link but cannot communicate.

## 1. Full (expanded) notation

| Device | Compressed | Expanded |
| --- | --- | --- |
| A | 2001:db8::ff00:42:8329 | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| B | 2001:db8:0:0:1::1 | 2001:0db8:0000:0000:0001:0000:0000:0001 |

In A, "::" replaces three zero groups. In B, "::" replaces two zero groups (groups 6 and 7).

## 2. Network prefix (first 64 bits = first 4 groups)

- Device A: 2001:0db8:0000:0000::/64 = 2001:db8::/64

- Device B: 2001:0db8:0000:0000::/64 = 2001:db8::/64

## 3. Same subnet?

Yes. With a /64 mask both addresses give the identical prefix 2001:db8::/64, so they are in the same subnet. The fault is therefore not a different prefix length or a different network prefix.

## 4. Addressing or prefix mismatch

- Device B's interface ID is 0001:0000:0000:0001. The "1" in the fifth group sits inside the interface-ID half, not in the subnet-ID field. This looks like a typing slip: the intended address was almost certainly 2001:db8:0:1::1 (subnet ID 1). Device B is placed in the wrong place in the addressing plan, and its interface ID is non-standard and hard to track.

- Device A's interface ID is 0000:ff00:0042:8329, which is also arbitrary and follows no planned pattern.

- Since the prefixes already match, the remaining causes of failure must be checked on the devices: IPv6 not enabled or no link-local address, wrong VLAN or port, a duplicate address, or ICMPv6/ND being filtered. Verify with the commands below.

Router(config-if)# ipv6 address 2001:db8:0:1::1/64   ! gateway

Router(config)# ipv6 unicast-routing

PC> ipv6 config            ! check address, prefix, link-local

PC> ping 2001:db8:0:1::20   ! test reachability

Router# show ipv6 interface brief

Router# show ipv6 neighbors  ! confirm Neighbor Discovery works

## 5. Optimized addressing

| Device | Address | Prefix |
| --- | --- | --- |
| Router (gateway) | 2001:db8:0:1::1 | /64 |
| Device A | 2001:db8:0:1::10 | /64 |
| Device B | 2001:db8:0:1::20 | /64 |

All three share the prefix 2001:db8:0:1::/64, with a clear, sequential interface ID in the last group.

## 6. Available host addresses

A /64 leaves 128 − 64 = 64 interface-ID bits, so there are 2^64 = 18,446,744,073,709,551,616 addresses. IPv6 has no broadcast address, so every address is usable (the all-zeros subnet-router anycast address is conventionally kept aside, giving 2^64 − 1 for hosts).

Question 5: Routing table and longest-prefix match

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

## 1. Matching entry (longest-prefix match)

Destination 192.168.2.100 is compared against each route:

| Route | Match? | Prefix length |
| --- | --- | --- |
| 192.168.1.0/24 | No (third octet 2 ≠ 1) | – |
| 192.168.2.0/24 | Yes | /24 |
| Default (0.0.0.0/0) | Yes (matches everything) | /0 |

The longest prefix is /24, so the entry 192.168.2.0/24 wins over the default route.

## 2. Next-hop router selected

10.0.0.3

## 3. Does the destination belong to the intended subnet?

Yes. 192.168.2.0/24 covers 192.168.2.0 – 192.168.2.255 (hosts .1 – .254), so 192.168.2.100 is a valid host in it. The forwarding decision is correct as written.

## 4. Configuration errors

The lookup itself is correct, so the problem lies in the route data rather than in the matching:

- The next hops of the two /24 routes are likely swapped or wrong: 10.0.0.3 may not be the router that actually connects to 192.168.2.0/24 (it may serve 192.168.1.0/24, or be down or unreachable).

- Next hops are given without an exit interface and without verifying that 10.0.0.2, .3 and .1 are on a directly connected network, so the next hop may not resolve through ARP.

- 10.0.0.3 may have no return route to the source, or its own routing may send the packet away from 192.168.2.0/24.

- If 10.0.0.3 is down, the router still prefers this route over the default route, so traffic is silently dropped and never falls back to 10.0.0.1.

## 5. Optimized routing table

| Destination | Exit interface / next hop | Purpose |
| --- | --- | --- |
| 192.168.1.0/24 | 10.0.0.2 (verified) | Specific LAN 1 route |
| 192.168.2.0/24 | 10.0.0.3 or the corrected router (verified) | Specific LAN 2 route |
| 0.0.0.0/0 | 10.0.0.1 | Single default route to the upstream |
| 192.168.0.0/16 (optional) | Null0, high administrative distance | Discards unknown private traffic instead of sending it out to the Internet |

- Verify each next hop with a ping and with "show ip route" and "traceroute", and fix the swapped or wrong next hop.

- Add an exit interface (or use a recursive next hop on a connected network) so the route is removed automatically when the link fails.

- Keep specific /24 routes; the two networks cannot be summarised into one route, since they use different next hops. If both later share a next hop, replace them with 192.168.0.0/22 or a suitable summary.

- For larger networks, use a dynamic protocol such as OSPF so failures reroute automatically.

Router(config)# no ip route 192.168.2.0 255.255.255.0 10.0.0.3

Router(config)# ip route 192.168.2.0 255.255.255.0 <correct next hop>

Router(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1

Router# show ip route

Router# ping 192.168.2.100

Router# traceroute 192.168.2.100

## 6. Route used if no destination matches

The default route (0.0.0.0/0) via 10.0.0.1, because /0 is the least specific match and is used only when no other prefix matches.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |